Belvidere Park District

NOTICE OF BID

1.	Sealed bids for site prep and installation of 3,800 sqft of stamped concrete pathway for the Belvidere Park District will be received until 10:00 A.M. on September 20th, 2023, at which time they will be publicly opened and read aloud. The bid should be addressed to Jonathan Cudzewicz, Belvidere Park District, 1006 W Lincoln Ave Belvidere, IL 61008

Any bids received after this date and time will be returned unopened.

I.	Instructions to Bidders

- Bids shall be submitted in a sealed envelope and clearly marked:

SEALED BID

"Baltic Mill Stamped Concrete Pathway"

- The successful bidder will be selected based on the District's evaluation of each bidder, reference checks, and the final approval by the Park District Board of Commissioners.

The Park District reserves the right to reject any or all bids and to waive informalities to serve the best interest of the Park District. Such decisions shall be final and not subject to recourse.

- This is a prevailing wage project.

- It is the responsibility of the Bidder to review the area and verify square footage. The Park District measured the area at 3,800 sqft.

- No bids may be withdrawn after the official opening. All bids

submitted must be valid for a minimum period of sixty (90) days after

the date set for the bid opening.

- The bidder's signature on the Bid Offer Form will be construed as

acceptance of and willingness to comply with all provisions of the

acts of the General Assembly of the State of Illinois relating to wages

of laborers, sexual harassment, drug-free workplace, preference to citizens of the United States and residents of the State of Illinois, and discrimination and intimidation of employees. Provisions of said acts are hereby incorporated by reference and become a part of this proposal and specification.

- The Park District is not subject to Sales Tax, Federal Excise Tax, or

Illinois Retailers Occupational Tax.

- The Park District will remove the current pavers and edging.

II.	Scope of Work  The following tasks are included in the base bid:

- Remove the existing paver base and replace it with 6 inches of CA7 gravel. Regrade, shape, and compact the CA7 base layer.

- Cut a straight edge along limestone and install expansion joint material between limestone and new concrete.

- Install 2,732 sqft of 6 inches of 4,000 psi concrete with wire mesh.

Install 1,068 sqft of 4 inches of 4,000 psi concrete with wire mesh.

Also includes:

  - 1 stamped pattern

  - 1 integral color

  - Clear concrete release

  - High gloss sealer

  - Gripping material such as Shark Grip

- The Contractor shall exercise care so as not to damage existing grass, walks, or other Park District property during the performance of its contract. The Contractor shall repair or replace any damaged grass or material, should damage occur during construction. The Contractor shall be responsible for removing and disposing of all debris and packing material from the job site.

The following tasks are to be proposed as alternate items:

- Remove and haul away the existing paver blocks and edging.

- Restoration along new concrete includes:

- Removal of excess gravel

- Backfill with pulverized topsoil

- Apply seed and blanket.

III.	Insurance

The Contractor shall carry Liability Insurance with a minimum coverage of $1,000,000.00, including Umbrella covering in the 	following categories:

Personal Injury

Property Damage

Vehicles

Completed Operations

The Contractor shall also be covered by Workman's Compensation Insurance that meets Illinois limits.

Certificates of Insurance showing such coverage shall be furnished with the bid.

IV.	Safety

Contractor shall fence in the area during construction to prevent citizen use during construction.

V.	References

Contractor shall list the names, addresses, and phone numbers of at least five (5) references where work of a similar nature and scope has been performed.

VI.	Completion Date

Project shall be completed as soon as possible but no later than October 31st, 2023.

VII.	Payment

The Owner shall make final payment within thirty (30) days after completion and acceptance of the work. No payment will be made to the Contractor until final waivers of lien have been submitted, assuring the Park District that all materials, including play equipment and concrete used on this project, have been fully paid for.

- END OF SECTION

Bid form page 1

VIII.	Bid Offer Form

Having read all sections of the bid documents, plans, and equipment specifications and familiarized ourselves with the existing conditions that might affect the work, we are pleased to bid as follows on all the work associated with this project as described above.

Base Bid:

Includes all the work described in the Scope of Work section above.

$_      	_________________

Alternate Bid Items:

- Remove and haul away the existing paver blocks and edging.

$_____________________

2. 	Restoration along new concrete

$_____________________

Bid form Page 2

CONTRACTOR'S REFERENCES

Below are listed at least five (5) references where we have performed work of a similar nature and scope to that described in this bid.

1.	_________________________________

_________________________________

_________________________________

_________________________________

2.	_________________________________

_________________________________

_________________________________

_________________________________

3.	_________________________________

_________________________________

_________________________________

_________________________________

4.	_________________________________

_________________________________

_________________________________

_________________________________

5.	_________________________________

_________________________________

_________________________________

_________________________________

Bid form Page 3

DATED THIS _______________ day of _________________, 2023

____________________________

Company name of Bidder (print)		(a) Individual    (  )

(b) Partnership  (  )

(c) Corporation (  )

____________________________

Full name of Bidder (Signature)

____________________________

Full name of Bidder (Print)

____________________________

Official Title

____________________________

Street Address

____________________________			______________________

City		State		Zip					Phone Number

______________________

Fax Number

______________________

E-Mail